TECHNICAL REPORT: MAMMONTO BUILDING ENTERPRISE NETWORK INFRASTRUCTURE

Milestone 3 Final Architecture, Layer 2/3 Implementation, Security, & Verification Portfolio

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

1. Executive Summary

This technical report documents the design, configuration, implementation, and verification of the enterprise network infrastructure for the Mammonto Building (Taung site). The primary objective of this project is to deliver a robust, highly structured, scalable, and secure network environment capable of supporting segregated multi-departmental operations, dynamic host auto-configuration, wireless edge mobility, and centralized infrastructure management.

The completed architecture utilizes an IEEE 802.1Q Router-on-a-Stick deployment model anchored by a Cisco ISR 4331 Edge Router, a Cisco Catalyst Core Switch, and a Cisco Catalyst Access Switch. Through strict VLAN isolation (VLANs 10, 20, 30, 40, and 99), centralized IOS DHCP IP address allocation, Spanning-Tree PortFast edge port optimization, and dedicated management server segregation, the Mammonto Building infrastructure achieves seamless inter-VLAN routing while enforcing strict boundary security and zero-loss end-to-end reachability.

2. Architectural & Topographical Overview

The Mammonto Building physical and logical topology is organized into three distinct hierarchical computing tiers: Edge Routing, Distribution/Core Switching, and Access Layer Switching.

Table 2.1: Master VLAN Addressing & Subnet Allocation Schedule

| VLAN ID | VLAN Name | Subnet CIDR | Default Gateway | Description & Scope |
| --- | --- | --- | --- | --- |
| 10 | ADMIN | 10.32.10.0/24 | 10.32.10.1 | Executive & Administrative workstations |
| 20 | PROJECT | 10.32.20.0/24 | 10.32.20.1 | Engineering & Project teams (+8 Expansion staff) |
| 30 | FINANCE | 10.32.30.0/24 | 10.32.30.1 | Accounting & Financial records workstations |
| 40 | STAFF-WIFI | 10.32.40.0/24 | 10.32.40.1 | Wireless AP endpoints (Laptops, Tablets, Mobiles) |
| 99 | MANAGEMENT | 10.32.99.0/24 | 10.32.99.1 | Central Infrastructure & Management Server (Static) |

3. Layer 2 Switching & IEEE 802.1Q Trunking

Layer 2 segmentation guarantees that broadcast domains are contained within individual departments, preventing broadcast storms and unauthorized sniffer access across VLAN boundaries. 802.1Q trunking is configured on all inter-switch and switch-to-router links to carry tagged frames across the backbone.

• Trunk Links: Core Switch Gi0/1 (to Router Gi0/0/0) and Core Switch Gi0/2 (to Access Switch Gi0/1) operate in mode trunk, permitting VLANs 10, 20, 30, 40, and 99.
• Access Ports: Switch-Access FastEthernet interfaces 0/1–3 (VLAN 20), FastEthernet 0/4–5 (VLAN 30), FastEthernet 0/6 (VLAN 40 AP01). Core Switch FastEthernet 0/3 is dedicated to Management Server (VLAN 99).
• Spanning-Tree Optimization: PortFast is enabled on all access edge interfaces to immediately transition ports from blocking to forwarding mode, bypassing STP listening/learning states for immediate DHCP binding upon host boot.

4. Layer 3 Inter-VLAN Routing & Dynamic DHCP Services

Inter-VLAN communication is achieved through a Router-on-a-Stick architecture configured on Cisco ISR 4331 physical interface GigabitEthernet 0/0/0. Logical 802.1Q subinterfaces (Gi0/0/0.10 through Gi0/0/0.99) terminate each VLAN broadcast domain and provide default gateway services.

To automate endpoint IP provisioning, Cisco IOS DHCP pools are active on the router for VLANs 10, 20, 30, and 40. Static address exclusion ranges (ip dhcp excluded-address 10.32.x.1 10.32.x.10) preserve low-ip address space for subinterface gateways and static infrastructure devices.

5. Network Security, Infrastructure Hardening, & Management

System security is enforced across both physical and control planes:
1. Out-of-Band Management Segregation: Management Server (10.32.99.10) resides on isolated VLAN 99, preventing standard workstation access while allowing administrative monitoring.
2. Port Security & Unused Port Suppression: Unassigned FastEthernet ports on access and core switches are administratively shut down or placed in non-routed state to prevent unauthorized rogue device attachment.
3. NVRAM Configuration Persistence: All running configurations across Router-ISR4331, Core-Switch-01, and Switch-Access are committed to NVRAM via 'write memory' to ensure zero loss upon cold boot or power disruption.

6. Comprehensive Verification, Testing Results, & ICMP Logs

Full verification was executed inside Cisco Packet Tracer, testing Layer 1 physical link status, Layer 2 trunking tables, Layer 3 subinterfaces, DHCP auto-leasing, and cross-subnet ICMP ping reachability.

Table 6.1: Comprehensive Network Test & Verification Evidence Summary

| Test ID | Target / Command | Source Device | Result | Verification Objective |
| --- | --- | --- | --- | --- |
| TEST-01 | show vlan brief / trunk | Switch-Access | PASS (100%) | Confirms VLAN segregation (10,20,30,40,99) and 802.1Q trunking on Gi0/1. |
| TEST-02 | show ip interface brief | Router-ISR4331 | PASS (100%) | Confirms subinterfaces Gi0/0/0.10 through .99 are Status: up / Protocol: up. |
| TEST-03 | ipconfig /all | Project PC 01 | PASS (100%) | Confirms host dynamic lease: IP 10.32.20.x, Gateway 10.32.20.1 via IOS DHCP. |
| TEST-04 | ping 10.32.20.1 | Project PC 01 | PASS (0% Loss) | Confirms 100% ICMP reachability to local subinterface default gateway. |
| TEST-05 | ping 10.32.99.10 | Project PC 01 | PASS (0% Loss) | Confirms Inter-VLAN routing across trunk links to Management Server (10.32.99.10). |

7. Conclusion & Recommendation for Future Expansion

The Mammonto Building enterprise network infrastructure has been fully designed, configured, visually decorated, and verified. All requirements for Milestone 3 have been successfully fulfilled. The network architecture provides complete inter-VLAN reachability, dynamic host configuration, and dedicated management security.

For future expansion, it is recommended to introduce First Hop Redundancy Protocols (FHRP) such as HSRP/VRRP across dual distribution switches, alongside IEEE 802.1X port-based authentication on access switches for enhanced wireless and endpoint access control.