MILESTONE 1: CLIENT DESIGN REVIEW

Project ID: CMPG325-2026-075
Client ID: CI-075
Assigned Organisation: Mammonto Building Projects (Taung)
Industry: Construction
Course: CMPG 325 — Computer Networks

# 1. Client Requirements

## 1.1 Confirmed Requirements

The network is being designed for Mammonto Building Projects (Taung), which operates in the construction industry.

Cisco Packet Tracer will be used to design and test the proposed network.

The internal network will use the IPv4 address block 10.32.0.0/16.

One of the main technical requirements is to provide reliable wireless LAN coverage throughout the required indoor working areas.

Access point placement must be considered carefully so that users receive suitable wireless coverage without unnecessary equipment.

The building is heritage-listed, so no new cabling may be installed through the external walls.

The network must also support 8 additional staff members in the Project/Technical department without requiring a complete redesign or the creation of new subnets.

## 1.2 Network Design Objectives

Provide reliable wired connectivity for desktop computers in each department.

Provide secure and reliable indoor wireless connectivity for staff using laptops and other mobile devices.

Separate departments using VLANs so that network traffic is organised and broadcast traffic is reduced.

Improve network security by keeping the different departments logically separated.

Leave enough IP address and switch capacity for future growth.

Make the network easy to expand without having to redesign the existing addressing structure.

# 2. Physical Topology Plan

The physical design takes the heritage restrictions of the building into account. The aim is to keep the network practical while avoiding any cabling work through the external walls.

Main Wiring Closet: The main wiring closet will contain the Edge Router (ISR 4331/2911), Core Switch (Catalyst 3650), Access Switch (Catalyst 2960), and the Central Management Server.

Internal Cable Routing: Ethernet cables will be routed internally through drop ceilings and internal raceways. This avoids making any new cable penetrations through the external walls.

Wireless Access Points: AP-01 and AP-02 will be placed on internal corridor walls. Their positions will be used to provide wireless coverage to the main user work areas.

Administration Department: Access switch ports Fa0/2–Fa0/8 will be assigned to VLAN 10.

Project/Technical Department: Access switch ports Fa0/9–Fa0/18 will be assigned to VLAN 20. Ports Fa0/11–Fa0/18 are kept available for the additional 8 staff members.

Finance/Support Department: Access switch ports Fa0/19–Fa0/24 will be assigned to VLAN 30.

# 3. Logical Topology and Subnetting Plan

The logical design separates the different departments into VLANs. This creates separate broadcast domains and makes it easier to manage and secure the network.

[ EDGE ROUTER ]
(10.32.99.1 /24)
        |
   [TRUNK LINK]
        |
[ CORE SWITCH 3650 ]
        |
   [TRUNK LINK]
        |
[ ACCESS SWITCH 2960 ]
   |        |        |        |
VLAN 10   VLAN 20   VLAN 30   VLAN 40
ADMIN     PROJECT   FINANCE   STAFF-WIFI
10.32.10.0/24  10.32.20.0/24  10.32.30.0/24  10.32.40.0/24

# 4. IP Addressing Plan

The assigned address block for the network is 10.32.0.0/16. The following subnets will be used for the different VLANs.

| VLAN | Name | Purpose | Subnet Mask | Gateway | Usable Host Range | Broadcast | Growth Capacity |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | ADMIN | Administrative staff | 255.255.255.0 | 10.32.10.1 | 10.32.10.2 – 10.32.10.254 | 10.32.10.255 | 254 hosts |
| 20 | PROJECT | Technical/Engineering | 255.255.255.0 | 10.32.20.1 | 10.32.20.2 – 10.32.20.254 | 10.32.20.255 | 254 hosts (+8 staff) |
| 30 | FINANCE | Accounting & Support | 255.255.255.0 | 10.32.30.1 | 10.32.30.2 – 10.32.30.254 | 10.32.30.255 | 254 hosts |
| 40 | STAFF-WIFI | Mobile WLAN devices | 255.255.255.0 | 10.32.40.1 | 10.32.40.2 – 10.32.40.254 | 10.32.40.255 | 254 hosts |
| 99 | MANAGEMENT | Network switches/AP/Server | 255.255.255.0 | 10.32.99.1 | 10.32.99.2 – 10.32.99.254 | 10.32.99.255 | Reserved |

# 5. Client Change Request: 8 Additional Staff Members

The Project/Technical department needs to accommodate 8 additional staff members. The current design already provides enough capacity for this change, so a new subnet or complete network redesign is not required.

IP Capacity: VLAN 20 uses the 10.32.20.0/24 subnet, which provides 254 usable IP addresses. Adding 8 staff members will therefore not require the subnet to be changed.

Switch Port Capacity: Ports Fa0/11–Fa0/18 are reserved for the additional Project/Technical staff. These ports will already be configured for VLAN 20.

Easy Expansion: When the new staff members join, their desktop computers can be connected to the available network outlets linked to these reserved switch ports.

No Major Redesign: The existing VLAN structure, IP addressing plan, router configuration and switch design can remain in place.

# 6. Summary

The proposed design meets the main requirements of Mammonto Building Projects (Taung) while taking the building's heritage restrictions into account. The network uses VLANs to separate departments, a dedicated wireless VLAN for staff mobile devices, and /24 subnets that provide room for future growth. The Project/Technical department can also accommodate the requested 8 additional staff members without changing the current subnetting plan or carrying out a full network redesign.